1.Component Type: Please add the data.

| NAME | TYPE | Pin | Property |
| --- | --- | --- | --- |
| Raspberry Pi | Pi 4 | 40-pin GPIO header | 3.3V logic, 5V tolerant on some pin |
| Power Supply | Chilong 100W | 6 terminals | 12V @ 5A, 5V @ 3A, 9V @ 1A, GND (x2), SSR |
| Main Motor | DC Motor | 2 wires | 12V DC, controlled by relay, max 2A |
| Turn Motor | NEMA 17 Stepper | 4 wires | 12V, 1.5A per phase, A4988 drive |
| Push Motor | NEMA 17 Stepper | 4 wires | 12V, 1.5A per phase, A4988 drive |
| LED Array | 6x LEDs | 2 pins each | 5mm LED, 220Ω resistor, 20mA each |
| IR Sensors | 8x IR Obstacle Sensors | 3 pins each | VCC(5V), GND, Signal (active LOW) |
| Servo 180° | MG996 | 3 pins | 5V, GND, PWM signal (GPIO 12) |
| Servo 90° | MG996 | 3 pins | 5V, GND, PWM signal (GPIO 13) |
| Relay Module | 1-Channel 5V Relay | 6 pins | VCC(5V), GND, IN(GPIO 5), COM, NO, NC |

### 2. Raspberry Pi Program Setup

| Step | Action |
| --- | --- |
| 1 | Install Raspberry Pi OS (Bookworm) on SD card |
| 2 | Enable SSH: sudo raspi-config → Interface Options → SSH → Enable |
| 3 | Install dependencies: sudo apt update && sudo apt install python3-pip vlc |
| 4 | Install RPi.GPIO: sudo apt install python3-rpi.gpio |
| 5 | Copy project files to /home/pi/interactive_display/ |
| 6 | Install video files (.mp4) to /home/pi/videos/ |
| 7 | Make main.py executable: chmod +x main.py |
| 8 | Run on boot: add @reboot python3 /home/pi/interactive_display/main.py to crontab |
| 9 | Test each component individually before full system test |
| 10 | Configure video player: cvlc --fullscreen for display output |

### 3. Warnings - PCB Assembly

| Warning | Description |
| --- | --- |
| ⚠ Don't | Solder components with incorrect polarity (LEDs, capacitors, diodes have polarity) |
| ⚠ Don't | Overheat pads when soldering (max 300°C / 30 seconds per pad) |
| ⚠ Don't | Bridge adjacent pads with solder (use flux to prevent) |
| ⚠ Don't | Install ICs backwards (check pin 1 marking on PCB) |
| ⚠ Don't | Apply power before inspecting all solder joints under magnification |
| ✅ Use | Leaded solder (63/37) for better wetting on plated through-holes |
| ✅ Use | No-clean flux for better solder flow on small pads |
| ✅ Use | Socket headers for Raspberry Pi, A4988, and Relay module (easier replacement) |
| ✅ Use | Heat sink on A4988 drivers if running steppers continuously |
| ✅ Use | Multimeter to check for shorts between VCC and GND before powering on |